Evaluation of Arthrospira (Spirulina) platensis growth on cheese whey in the context of circular economy

Vasiliki Athanasiadou1*, Eleftheria E. Klontza1, Petros Dimitriou-Christidis2,3, Michalis Fountoulakis1 and Demetris F. Lekkas1

Rationale: Cultivation of microalgae using wastewater could be considered as a cost-effective treatment method, while providing the necessary nutrients to for the production of high-value biomass. The potential use of cheese whey for Arthrospira (spirulina) platensis cultivation could provide a sustainable solution aligned with circular economy principles for the treatment of dairy industry wastewater, in regions such as North Aegean (Greece).

Aim: The aim of this research is to investigate the parameters that influence the growth of Arthrospira platensis biomass on whey rich solutions, as a substitute to chemical nutrient feeds, according to the practices of circular economy and sustainable development.

Methods: Specifically, the effect of `different whey dilution ratios and irradiation conditions on biomass productivity, nutrient consumption and COD removal were examined in batch experiments.

Results: show that the concentration of 10% (v/v) cheese whey exhibited the best performance in both experimental illumination sets; continuous and light/dark cycles while providing the necessary nutrients to for the production of high-value biomass.

Conclusions & Outlook: The potential use of cheese whey for Arthrospira (spirulina) platensis cultivation could provide a sustainable solution aligned with circular economy principles for the treatment of dairy industry wastewater, in regions such as North Aegean (Greece).

1Waste Management Laboratory, Department of Environment, University of the Aegean, University Hill, 81100 Mytilene, Greece

2Earth Space Technical Ecosystem Enterprises SA

3Independent environmental consultant

*Corresponding author - email: envm21041@env.aegean.gr

- Acknowledgements: We acknowledge support of this work by the project “Center of Sustainable and Circular Bioeconomy [Aegean_BIOECONOMY]” (MIS 5045851) which is implemented under the Action “Reinforcement of the Research and Innovation Infrastructure”, funded by the Operational Programme "Competitiveness, Entrepreneurship and Innovation" (NSRF 2014-2020) and co-financed by Greece and the European Union (European Regional Development Fund).

- Data availability: We acknowledge support of this work by the project “Center of Sustainable and Circular Bioeconomy [Aegean_BIOECONOMY]” (MIS 5045851) which is implemented under the Action “Reinforcement of the Research and Innovation Infrastructure”, funded by the Operational Programme "Competitiveness, Entrepreneurship and Innovation" (NSRF 2014-2020) and co-financed by Greece and the European Union (European Regional Development Fund).

- Funding: We acknowledge support of this work by the project “Center of Sustainable and Circular Bioeconomy [Aegean_BIOECONOMY]” (MIS 5045851) which is implemented under the Action “Reinforcement of the Research and Innovation Infrastructure”, funded by the Operational Programme "Competitiveness, Entrepreneurship and Innovation" (NSRF 2014-2020) and co-financed by Greece and the European Union (European Regional Development Fund).

- Author Credits: We acknowledge support of this work by the project “Center of Sustainable and Circular Bioeconomy [Aegean_BIOECONOMY]” (MIS 5045851) which is implemented under the Action “Reinforcement of the Research and Innovation Infrastructure”, funded by the Operational Programme "Competitiveness, Entrepreneurship and Innovation" (NSRF 2014-2020) and co-financed by Greece and the European Union (European Regional Development Fund).

References

[1] Aguado J., Arsuaga J.M., Arencibia A., Lindo M. and Gascón V. (2009), Aqueous heavy metals removal by adsorption on amine-functionalized mesoporous silica, Journal of Hazardous Materials, 163, 213-221.

[2] Allen S.J., McKay G. and Khader K.Y.H. (1989), Intraparticle diffusion of a basic dye during adsorption onto Sphagnum Peat, Environmental Pollution, 56, 39-50.

[3] Areco M.M. and Afonso M.S. (2010), Copper, zinc, cadmium and lead biosorption by Gymnogongrus torulosus. Thermodynamics and kinetics studies, Colloids and Surfaces B: Biointerfaces, 81, 620-628.

[4] Axtell N.R., Sternberg S.P.K. and Claussen K. (2003), Lead and nickel removal using Microspora and Lemna minor, Bioresource Technology, 89, 41-48.

[5] Bhattacharya A.K., Mandal S.N., and Das S.K. (2006), Adsorption of Zn (II) from aqueous solution by using different adsorbents, Chemical Engineering Journal, 123, 43-51.

[6] Bulut Y. and Tez Z. (2007), Adsorption studies on ground shells of hazelnut and almond, Journal of Hazardous Materials, 149, 35-41.

[7] Çalışkan N., Kul A.R., Alkan S., Gogut E.G. and Alacabey I. (2011), Adsorption of zinc (II) on diatomite and manganese-oxide-modified diatomite: A kinetic and equilibrium study, Journal of Hazardous Materials, 193, 27-36.

[8] Cheremisinoff P.N. (1995), Handbook of Water and Wastewater Treatment Technology, Marcel Dekker Inc., New York.